1998/1/17

Elution positions of 252 Pyridylaminated (PA) N-glycans, using Shimpack CLC-ODS and Amide-80 columns. *, 75 structures confirmed by NMR measurements. Abbreviations: EPO, Erythropoietin; HUK, Human urinary kallikrein; PPK, porcine pancreatic kallikrein (19).

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 000.1 | Mana6 ¦ ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Mana3 | 7.3 7.4 | 4.1 4.3 | Duck ovomucoid,A Saposin A&C | 1 9 11 |
| 010.0* | Mana6 ¦ Fuca6 ¦ ¦ Manb4GlcNAcb4GlcNAc | 10.1 | 3.5 | Saposin A,g2 | 11 |
| 010.1* | Mana6 ¦ Fuca6 ¦ ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Mana3 | 9.7 10.2 | 4.7 | Fc-receptor Saposin A,g1 | 1 11 17 |
| 100.1* | Mana6 ¦ ¦ GlcNAcb2 ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Mana3 | 9.3 9.5 | 4.7 | Saposin C Fc-receptor Duck ovomucoid,G | 1 9 11 17 |
| 100.2 | Mana6 ¦ ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Mana3 ¦ GlcNAcb2 | 7.7 7.4 | 4.7 | unseparable from 000.1 | 1 |
| 100.3 | Mana6 ¦ ¦ Galb4GlcNAcb2 ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Mana3 | 10.0 9.9 | 5.4 5.6 | Duck ovomucoid,H1 | 1 9 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 100.4 | Mana6 ¦ ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Mana3 ¦ Galb4GlcNAcb2 | 8.1 8.0 | 5.6 5.7 | Duck ovomucoid,C | 1 9 |
| 100.6 | GlcNAcb6 ¦ Mana6 ¦ ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Mana3 | 7.7 | 4.8 | “100.8”-G |  |
| 100.7 | Mana6 ¦ ¦ Manb4GlcNAcbbGlcNAc ¦ ¦ GlcNAcb4Mana3 | 8.3 | 4.5 | Duck ovomucoid,D Saposin C,q | 9 11 |
| 100.8 | Galb4GlcNAcb6 ¦ Mana6 ¦ ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Mana3 | 7.8 | 5.8 | “110.8”-F |  |
| 100.9 | Mana6 ¦ ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Galb4GlcNAcb4Mana3 | 8.7 8.5 | 5.6 5.8 | Duck ovomucoid,E | 9 |
| 100.10 | Mana6 ¦ ¦ Manb4GlcNAcb4GlcNAc ¦ ¦ Galb3GlcNAcb4Mana3 | 8.5 | 5.3 | “1A5-300.22”-G-GN-S |  |
| 101.2 | Mana6 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Mana3 ¦ GlcNAc2 | 10.4 | 5.1 | Duck ovomucoid,I | 1 9 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 101.4 | Mana6 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Mana3 ¦ Gal4GlcNAc2 | 11.0 | 5.8 | “201.3”-GN | 1 |
| 110.1 | Mana6 ¦ ¦ Fuca6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Mana3 | 12.7 | 5.1 | “110.3”-G | 1 17 |
| 110.2 | Mana6 ¦ Fuca6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Mana3 ¦ GlcNAc2 | 10.2 | 5.1 | IgG, Q | 1 |
| 110.3 | Mana6 ¦ ¦ Fuca6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Mana3 | 13.0 | 6.1 | “210.2”-GN | 1 17 |
| 110.4 | Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 10.9 | 6.3 | IgG, R Saposin A&C,h |  |
| 110.5* | Man6 ¦ ¦ Fuc6 Gal3Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 13.7 | 6.9 | “210.21”-GN |  |
| 110.6 | GlcNAc6 ¦ Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 10.3 | 5.3 | “110.8”-G |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 110.7 | Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 | 11.0 | 5.0 | “110.9”-G |  |
| 110.8 | Gal4GlcNAc6 ¦ Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 10.5 | 6.2 | “410.4”-GN |  |
| 110.9 | Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc Gal4GlcNAc4 ¦ ¦ ¦ Man3 | 11.4 | 6.2 | “410.3”-GN |  |
| 111.2 | Man6 ¦ Fuc6 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 14.3 | 5.5 | “111.4”-G Saposin C,u | 1 11 |
| 111.4 | Man6 ¦ Fuc6 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 15.5 | 6.4 | “211.3”-GN | 1 |
| 200.1* | GlcNAc2Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 8.9 | 5.1 | IgG,A | 1 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 200.2* | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 9.4 | 5.9 | IgG,B | 1 |
| 200.3* | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 9.6 | 6.1 | IgG,C | 1 |
| 200.4* | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gab4GlcNAc2 | 10.2 | 7.0 | IgG,D | 1 |
| 200.9 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 9.0 | 5.0 | “200.10”-G |  |
| 200.10 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 9.8 | 5.8 | “1A1-200.10”-S |  |
| 200.11 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 7.3 | 7.3 | “210.11”-F |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 200.12 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 8.5 | 7.3 | “210.12”-F |  |
| 200.13 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 10.2 10.3 | 7.0 7.2 | “210.13”-F |  |
| 200.14 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 |  |  | “210.14”-F |  |
| 200.15 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 | 11.0 | 6.9 | “2A5-300.8”-G-GN-S |  |
| 200.17 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 | 10.1 | 6.7 | “2A6-300.22”-G-GN-S |  |
| 200.18 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal3 | 11.0 | 6.6 | “2A5-300.22”-G-GN-S |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 200.22 | Gal4GlcNAc2Man6 ¦ ¦ Gal3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc2Man3 | 10.3 | 7.5 |  | 1 |
| 200.23 | Gal4GlcNAc2Man6 ¦ ¦ Gal3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc2Man3 ¦ Gal3 | 10.6 | 8.6 |  | 1 |
| 201.1 | Man6 ¦ ¦ GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 12.3 | 5.4 | IgG,I | 1 |
| 201.2 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 13.3 | 6.2 | IgG,J | 1 |
| 201.3 | Man6 ¦ ¦ GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 13.7 | 6.3 | IgG,K | 1 |
| 201.4 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 14.5 | 7.2 | IgG,L | 1 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 201.15* | Man6 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 14.8 | 5.3 | Duck ovomucoid,L Saposin C,w | 9 11 |
| 201.16 | Man6 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 15.3 | 6.3 | Duck ovomucoid,M | 9 |
| 210.1* | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 11.8 12.3 | 5.5 | IgG,E | 1 |
| 210.2* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 12.4 13.1 | 6.3 | IgG,F | 1 |
| 210.3* | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 12.7 13.3 | 6.4 | IgG,G | 1 |
| 210.4* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 13.8 14.1 | 7.4 | IgG,H EPO,E | 1 2 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 210.11 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 10.1 | 7.7 | “410.9 (PPK,H-2)”-GN | 1 19 |
| 210.12 | Gal4GlcNAc6 ¦ Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 11.6 | 7.5 | “410.7 (PPK,K-1)”-GN | 1 19 |
| 210.13 | Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 13.9 | 7.5 | “410.8 (PPK,L-1)”-GN | 1 19 |
| 210.14 | Gal4GlcNAc6 ¦ Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 | 12.0 | 7.7 | “410.16b (HUK,q)”-GN | 13 |
| 210.15 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 | 14.8 | 7.6 | “310.8a (HUK,t)”-GN | 13 |
| 210.16 | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 10.1 | 5.5 | “210.11”-G |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 210.21 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Gal3 ¦ ¦ Man3 ¦ GlcNAc2 | 13.1 | 7.1 | “210.22”-G | 1 |
| 210.22* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Gal3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 14.1 | 8.1 | IgG,T Acetylcholine receptor,R | 1 10 |
| 210.23* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Gal3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Gal3 | 14.4 | 9.0 | IgG,U Acetylcholine receptor,S | 1 10 |
| 211.1* | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 17.8 | 5.8 | IgG,M | 1 |
| 211.2* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 18.7 | 6.6 | IgG,N | 1 |
| 211.3* | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 18.9 | 6.7 | IgG,O | 1 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 211.4* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 20.2 | 7.5 | IgG,P | 1 |
| 300.1* | Man6 ¦ ¦ GlcNAc2 Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 10.9 | 5.5 | 1-acid glycoprotein | 1 19 |
| 300.2 | Man6 ¦ ¦ Gal4GlcNAc2 Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 10.8 11.1 | 6.0 6.2 | “310.2”-F |  |
| 300.3 | Man6 ¦ ¦ GlcNAc2 Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 11.8 | 6.3 | “310.3”-F |  |
| 300.4 | Man6 ¦ ¦ GlcNAc2 Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 11.9 | 6.2 | “1A5-300.8”-G-S unseparable from 300.2 |  |
| 300.5 | Man6 ¦ ¦ Gal4GlcNAc2 Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 12.3 12.0 | 7.3 | Acetylcholine receptor,K | 10 |
| 300.6 | Man6 ¦ ¦ Gal4GlcNAc2 Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 11.3 | 7.0 | “2A5-300.8”-G-S |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 300.8* | Man6 ¦ ¦ Gal4GlcNAc2 Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 12.7 | 8.3 | Fetuin Acetylcholine receptor,M 1-acid glycoprotein | 1 10 19 |
| 300.9 | Man6 ¦ ¦ GlcNAc2 Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 12.0 | 7.1 | “2A6-300.8”-G-S |  |
| 300.11 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 7.3 | 5.8 |  | 1 |
| 300.12 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 |  |  |  |  |
| 300.13 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 8.2 | 7.2 | “2A1-300.18(ntegrin)”-G-S | 8 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 300.14 | GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 7.9 | 7.3 | “2A3-300.18(Integrin)”-G-S | 8 |
| 300.15 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 7.5 | 7.2 | “2A2-300.18(Integrin)”-G-S | 8 |
| 300.18 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 8.1 | 8.6 |  | 1 |
| 300.19 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 | 8.2 | 8.5 | “3A5-410.16(Integrin)”-F-G-GN-S | 8 |
| 300.20 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 10.7 | 8.3 | “3A3-410.16(Integrin)”-F-G-GN-S | 8 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 300.21 | GlcNAc2Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal3GlcNAc4Man3 ¦ GlcNAc2 | 11.1 11.6 | 6.0 6.2 |  | 1 |
| 300.22* | Gal4GlcNAc2Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal3GlcNAc4Man3 ¦ Gal4GlcNAc2 | 13.2 | 8.2 | Fetuin Acetylcholine receptor,N | 1 10 |
| 300.23 | GlcNAc2Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal3GlcNAc4Man3 ¦ Gal4GlcNAc2 | 12.3 | 7.0 | “2A6-300.22”-G-S |  |
| 300.24 | Gal4GlcNAc2Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal3GlcNAc4Man3 ¦ GlcNAc2 | 11.5 | 6.9 | “2A5-300.22”-G-S |  |
| 300.31 | Gal4GlcNAc2Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Fuc3 ¦ Gal4GlcNAc2 | 13.0 12.8 | 8.7 9.0 | 1-acid glycoprotein | 1 |
| 301.1* | Man6 ¦ ¦ GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 16.9 | 5.7 | Duck ovomucoid,N Acetylcholine receptor,T Saposin C,x Ovalbumin,J | 9 10 11 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 301.2* | Man6 ¦ ¦ GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 17.5 | 6.7 | Duck ovomucoid,O Acetylcholine receptor,U Duck ovomucoid Ovalbumin,K | 9 10 |
| 301.3* | Man6 ¦ ¦ GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 19.4 | 7.5 | Acetylcholine receptor,V | 10 |
| 301.4* | Man6 ¦ ¦ Gal4GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 20.2 | 8.2 | Acetylcholine receptor,W | 10 |
| 310.1* | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 14.8 | 5.8 | Saposin A,n PPK,M | 1 11 19 |
| 310.2* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 15.7 | 6.8 | PPK,N | 19 |
| 310.3* | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 16.6 | 6.9 | PPK,O | 19 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 310.5* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 17.2 | 7.8 | PPK,P | 19 |
| 310.8* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 16.9 17.7 | 8.7 | EPO,J PPK,Q | 2 19 |
| 310.9 | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 |  |  |  |  |
| 310.11 | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 10.3 | 6.4 |  | 1 |
| 310.12 | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 |  |  |  |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 310.13 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 10.9 | 7.6 | “2A1-310.18(Integrin)”-G-S | 8 |
| 310.14 | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 10.6 | 7.7 | “2A3-310.18(Integrin)”-G-S |  |
| 310.15 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 10.1 | 7.6 | “2A2-310.18(Integrin)”-G-S |  |
| 310.18* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 10.7 11.2 | 9.0 | EPO, C Saposin A&C,i | 2 11 |
| 310.19 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 | 11.4 10.8 | 9.2 8.8 | “410.16a(HUK)”-GN | 13 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 310.20 | Gal4GlcNAc6 ¦ Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 13.8 | 8.6 | “3A3-410.16(Integrin)”-G-GN-S | 8 |
| 310.21 | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ GlcNAc2 | 15.5 | 6.9 | “310.22”-G | 1 |
| 310.22 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 | 18.0 | 8.6 | EPO,K | 2 |
| 310.31 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Fuc3 ¦ Gal4GlcNAc2 | 16.6 | 9.2 | “3A2-310.31”-S a little before Of 310.8 |  |
| 310.37* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Gal3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 ¦ Gal3 | 18.8 | 11.2 | Whale Thyroglobulin |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 310.48 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Gal3 ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Gal3 ¦ Gal4GlcNAc2 ¦ Gal3 | 19.7 | 11.0 | Whale Thyroglobulin | 1 |
| 400.1* | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 9.0 | 6.3 | 1-acid glycoprotein |  |
| 400.2 | GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 8.7 9.0 | 6.8 6.9 | “410.2”-F |  |
| 400.3 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 9.3 9.5 | 6.9 | “410.3”-F |  |
| 400.5 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 9.9 10.1 | 6.9 7.0 | “410.5”-F |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 400.6 | GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 9.7 9.9 | 7.9 | “410.6”-F |  |
| 400.7 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 10.2 | 7.9 | “410.7”-F |  |
| 400.8 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 10.4 10.5 | 7.9 8.1 | “410.8”-F |  |
| 400.9 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 9.3 | 7.9 | “410.9”-F |  |
| 400.10 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 |  |  |  |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 400.11 | GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 |  |  |  |  |
| 400.12 | GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 10.1 10.2 | 8.8 9.0 | “410.12”-F |  |
| 400.13 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 10.0 | 8.8 | “410.13”-F |  |
| 400.14 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 10.7 | 8.6 | “410.14”-F |  |
| 400.15 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 9.8 | 8.6 | “410.15”-F |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 400.16* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 10.2 10.6 | 9.8 9.9 | 1-acid glycoprotein | 1 |
| 400.21 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ GlcNAc2 | 10.1 | 7.0 | “410.21”-F | 1 |
| 400.22 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 | 10.7 | 9.7 | “410.22”-F | 1 |
| 400.31 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Fuc3 ¦ Gal4GlcNAc2 | 10.5 | 10.5 | 1-acid glycoprotein | 1 |
| 400.41 | GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc GlcNAc3 ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 9.9 | 7.2 | “410.41”-F | 1 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 400.42 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Gal4GlcNAc3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 11.9 | 11.2 | “410.42”-F | 1 |
| 400.43 | Gal4GlcNAc6 ¦ ¦ GlcNAc3 Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 10.5 | 7.2 | “410.43”-F | 1 |
| 400.44 | Gal4GlcNAc6 ¦ ¦ Gal4GlcNAc3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 12.2 | 11.1 | “410.44”-F | 1 |
| 400.51 | Gal4GlcNAc6 ¦ ¦ GlcNAc3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc GlcNAc3 ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 11.6 | 8.8 | “410.51”-F | 1 |
| 400.52 | Gal4GlcNAc6 ¦ ¦ Gal4GlcNAc3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Gal4GlcNAc3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 13.5 | 12.4 | “410.52”-F | 1 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 400.61 | Gal4GlcNAc6 ¦ ¦ GlcNAc3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc GlcNAc3 ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc3Gal4GlcNAc2 | 14.0 | 9.8 | “410.61”-F | 1 |
| 400.62 | Gal4GlcNAc6 ¦ ¦ Gal4GlcNAc3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Gal4GlcNAc3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Gal4GlcNAc3 | 15.7 | 13.6 | “410.62”-F | 1 |
| 401.1 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 15.1 | 6.4 | “401.16”-G |  |
| 401.16 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 18.6 | 10.0 | “411.16”-F |  |
| 410.1* | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 12.4 | 6.6 | PPK,G | 19 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 410.2* | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 12.3 | 7.5 | PPK,F | 19 |
| 410.3* | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 12.8 | 7.5 | PPK,H-1 | 19 |
| 410.4* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 12.8 | 7.5 | PPK,H-1' | 19 |
| 410.5* | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 13.8 | 7.6 | PPK,J-1 | 19 |
| 410.6* | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 13.6 | 8.4 | PPK,I | 19 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 410.7* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 14.1 | 8.5 | PPK,K-1 | 19 |
| 410.8* | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 14.4 | 8.5 | PPK,L-1 | 19 |
| 410.9* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 12.8 | 8.4 | PPK,H-2 | 19 |
| 410.12* | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 14.1 | 9.5 | PPK,K-2 | 19 |
| 410.13* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 | 13.8 | 9.3 | PPK,J-2 | 19 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 410.14 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 13.7 | 9.2 8.9 | “3A3-410.16(Integrin)”-G-S | 8 |
| 410.15 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 12.5 | 8.9 | “3A5-410.16(Integrin)”-G-S | 8 |
| 410.16* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 14.2 | 10.3 | EPO,F PPK,L-2 | 2 19 |
| 410.21 | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ GlcNAc2 | 13.3 | 7.4 | “410.22”-G | 1 |
| 410.22* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 | 14.4 | 10.2 | EPO,G | 2 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 410.41 | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc GlcNAc3 ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 13.2 | 7.7 | “410.42”-G | 2 |
| 410.42* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Gal4GlcNAc3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 16.2 | 11.6 | EPO,H | 2 |
| 410.43 | Gal4GlcNAc6 ¦ ¦ GlcNAc3 Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 14.1 | 7.8 | “410.44”-G | 2 |
| 410.44* | Gal4GlcNAc6 ¦ ¦ Gal4GlcNAc3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 16.3 | 11.6 | EPO,I | 2 |
| 410.51 | Gal4GlcNAc6 ¦ ¦ GlcNAc3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc GlcNAc3 ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 15.8 | 9.0 | “410.52”-G | 2 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 410.52* | Gal4GlcNAc6 ¦ ¦ Gal4GlcNAc3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Gal4GlcNAc3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 18.3 | 12.6 | EPO,L | 2 |
| 410.61 | Gal4GlcNAc6 ¦ ¦ GlcNAc3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc GlcNAc3 ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ GlcNAc3 | 18.8 | 10.0 | “410.62”-G | 2 |
| 410.62 | Gal4GlcNAc6 ¦ ¦ Gal4GlcNAc3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Gal4GlcNAc3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Gal4GlcNAc3 | 20.8 | 13.8 | EPO,M | 2 |
| 411.16* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 25.5 | 10.2 | Peanut lectin receptor |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 411.17* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Fuc2 | 28.0 | 10.6 | Peanut lectin receptor |  |
| 501.1* | GlcNAc6 ¦ GlcNAc4Man6 ¦ ¦ GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 14.8 | 6.9 | Chicken ovomucoid | 1 |
| 501.2* | GlcNAc6 ¦ GlcNAc4Man6 ¦ ¦ GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 15.7 15.3 | 7.4 8.0 | Chicken ovomucoid | 1 |

High-mannose (17)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| M1.1 | Man4GlcNAc4GlcNAc | 6.3 6.6 | 2.3 2.0 |  | 1 |
| M2.1* | Man6 ¦ ¦ Man4GlcNAc4GlcNAc | 7.2 7.4 | 3.1 3.3 | Duck ovomucoid,B | 1 9 |
| M3.3 | Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc | 7.2 7.5 | 4.1 | Saposin C,p | 11 12 |
| M4.1* | Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ Man3 | 7.5 | 5.1 | from Dr. Hase, S. Saposin A&C,b2 | 11 |
| M4.7 | Man6 ¦ Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc | 7.3 | 5.1 |  | 12 |
| M4.11 | Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 7.4 | 5.3 |  | 4 |
| M5.1* | Man6 ¦ Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 7.2 | 6.2 | Ovalbumin,C Acetylcholine receptor,D | 1 10 |
| M5.11 | Man6 ¦ Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 7.3 | 6.5 |  | 4 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| M6.1* | Man6 ¦ Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Man2 | 6.1 | 7.1 | Ovalbumin,B Acetylcholine receptor,C | 1 10 |
| M6.10 | Man6 ¦ Man6 ¦ ¦ Man3 ¦ ¦ Man4GlcNAc4GlcNAc Man2 ¦ ¦ Man3 | 7.9 | 6.8 |  | 12 |
| M7.1 | Man6 ¦ Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Man2 ¦ Man2 | 5.8 | 8.0 | Pig thyroglobulin | 1 |
| M7.2 | Man6 ¦ ¦ Man2 Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Man2 | 5.1 | 8.1 | Ovalbumin (Quail, Chick) Pig thyroglobulin | 1 |
| M7.7 | Man6 ¦ Man6 ¦ ¦ Man3 ¦ ¦ Man4GlcNAc4GlcNAc Man2 ¦ ¦ Man3 ¦ Man2 | 6.8 | 7.7 |  | 12 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| M8.1* | Man6 ¦ ¦ Man2 Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Man2 ¦ Man2 | 4.9 | 9.0 | Acetylcholine receptor,A | 1 10 |
| M8.2 | Man6 ¦ Man6 ¦ ¦ Man3 ¦ ¦ Man4GlcNAc4GlcNAc Man2 ¦ ¦ Man3 ¦ Man2 ¦ Man2 | 6.4 | 8.5 |  | 12 |
| M9.1* | Man6 ¦ ¦ Man2 Man6 ¦ ¦ Man3 ¦ ¦ Man4GlcNAc4GlcNAc Man2 ¦ ¦ Man3 ¦ Man2 ¦ Man2 | 5.2 | 9.7 | Acetylcholine receptor,B | 1 10 |
| M9.2* | Man6 ¦ ¦ Man2 Man6 ¦ ¦ Man3 ¦ ¦ Man4GlcNAc4GlcNAc Man2 ¦ ¦ Man3 ¦ Man2 ¦ Man2 ¦ Glc3 | 6.3 | 10.3 |  | 1 |

Hybrid (6)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| H4.1 | Man6 ¦ ¦ Man3 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 9.5 | 5.9 | Ovalbumin, E | 1 |
| H4.3 | Man6 ¦ ¦ Man3 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 13.1 | 6.2 | Ovalbumin,H | 1 |
| H4.4 | Man6 ¦ ¦ Man3 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 13.6 | 7.2 | Ovalbumin,I | 1 |
| H5.1 | Man6 ¦ Man6 ¦ ¦ Man3 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 8.7 | 6.9 | Ovalbumin,D | 1 |
| H5.3 | Man6 ¦ Man6 ¦ ¦ Man3 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 11.9 | 7.2 | Ovalbumin,F | 1 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| H5.4 | Man6 ¦ Man6 ¦ ¦ Man3 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 12.4 | 8.2 | Ovalbumin,G2 | 1 |

Plant, Insect (30)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 000.1F* | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ Fuc3 Man3 | 5.6 5.8 | 5.5 | Bermuda grass,B | 3 |
| 000.1X* | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ Xyl2 Man3 | 7.2 | 4.6 4.8 | Amylase,e1 | 2 |
| 000.1FX* | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 | 5.6 | 5.6 5.7 | Laccase,a Miraculin,A | 2 |
| 100.1X | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ Xyl2 Man3 | 8.7 | 5.1 | Amylase,g1 | 2 |
| 100.1FX* | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 | 7.1 7.3 | 6.2 | Laccase,b2 Bermuda grass,E | 2 3 |
| 100.2F | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ Fuc3 Man3 ¦ GlcNAc2 | 6.1 | 5.5 |  | 4 |
| 100.2X | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ Xyl2 Man3 ¦ GlcNAc2 | 7.7 | 5.1 | Amylase,f | 2 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 100.2FX* | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 ¦ GlcNAc2 | 6.0 5.8 | 6.4 6.2 | Laccase,b1 | 2 |
| 100.3FX | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 | 7.3 | 7.0 | “200.2FX”-GN | 2 |
| 100.4FX | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 ¦ Gal4GlcNAc2 | 6.3 | 6.9 | “100.31FX”-F | 2 |
| 100.31FX* | Fuc6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 | 7.4 | 8.0 | Miraculin,D | 2 |
| 100.41FX* | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Fuc6 Man3 ¦ ¦ Gal4GlcNAc2 | 6.3 | 8.0 | Miraculin,B | 2 |
| 200.1F | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ Fuc3 Man3 ¦ GlcNAc2 | 7.4 | 5.9 |  | 4 |
| 200.1X* | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ Xyl2 Man3 ¦ GlcNAc2 | 8.9 | 5.5 | Amylase,g2 | 2 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 200.1FX* | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 ¦ GlcNAc2 | 7.0 | 6.5 | Laccase,c | 2 |
| 200.2FX | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 ¦ GlcNAc2 | 6.9 | 7.2 | “200.24FX”-F | 2 |
| 200.4FX | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 ¦ Gal4GlcNAc2 | 7.1 | 8.1 | “210.41FX”-F | 2 |
| 200.24FX* | Fuc6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Man3 ¦ GlcNAc2 | 7.2 | 8.3 | Laccase,e | 2 |
| 210.41FX* | Fuc6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ ¦ Xyl2 Fuc3 Fuc6 Man3 ¦ ¦ Gal4GlcNAc2 | 7.7 | 10.3 | Laccase,f Miraculin,E | 2 |
| M1.1F | Man4GlcNAc4GlcNAc ¦ Fuc3 | 5.2 5.3 | 3.1 3.0 | “M1.1FX”-X | 7 |
| M1.1X | Man4GlcNAc4GlcNAc ¦ Xyl2 | 7.3 | 2.3 | “M2.1X”-M | 2 |
| M1.1FX | Man4GlcNAc4GlcNAc ¦ ¦ Xyl2 Fuc3 | 6.1 6.3 | 3.4 3.2 | “M2.1FX”-M | 2 3 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| M2.1F | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ Fuc3 | 6.1 | 4.2 | “M2.1FX”-X | 2 3 |
| M2.1X | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ Xyl2 | 8.6 8.8 | 3.2 3.3 | “100.1X”-M-GN | 2 |
| M2.1FX | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Xyl2 Fuc3 | 6.9 7.3 | 4.1 4.4 | Bromelain Miraculin,C | 2 |
| M2.2F | Man4GlcNAc4GlcNAc ¦ ¦ ¦ Fuc3 Man3 | 4.9 5.2 | 4.2 | Bermuda grass,A | 3 |
| M3.3F | Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ Fuc3 | 5.9 | 5.1 | Horse radish peroxidase,e | 6 |
| M3.3FF | Man6 ¦ ¦ Fuc6 Man3 ¦ ¦ Man4GlcNAc4GlcNAc ¦ Fuc3 | 8.5 | 5.5 | Insect IgG,G | 5 |
| M5.11F | Man6 ¦ Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ Fuc3 Man3 ¦ GlcNAc2 | 6.0 | 7.3 | Sycamore cell | 4 |
| M5.11X | Man6 ¦ Man6 ¦ ¦ Man3 ¦ Man4GlcNAc4GlcNAc ¦ ¦ ¦ Xyl2 Man3 ¦ GlcNAc2 | 7.4 | 6.7 | Sycamore cell | 4 |

GalNAc-containing complex-type. (42)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 100.4a | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 8.1 | 5.2 |  |  |
| 110.4a | Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 10.9 | 5.7 |  |  |
| 200.3a | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 9.7 | 5.4 |  |  |
| 200.4a | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 10.2 | 6.4 | HUK,b |  |
| 200.4a1 | Man6 ¦ ¦ GalNAc4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 11.3 | 6.5 | “1A2-210.4a1(Fc-receptor)”-S-F | 17 |
| 200.4b | Man6 ¦ ¦ GalNAc4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 10.9 | 6.2 | HUK,f |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 210.3a | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 13.0 | 6.0 | HUK,l |  |
| 210.4a* | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 13.6 | 6.9 | HUK,m |  |
| 210.4a1 | Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 15.5 | 6.9 | “1A2-210.4a1(Fc-receptor)”-S | 17 |
| 210.4b* | Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 14.7 | 6.7 | HUK,p |  |
| 300.3a | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 12.0 | 5.7 |  |  |
| 300.5b | Man6 ¦ ¦ GalNAc4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 13.7 | 6.4 |  |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 300.8a | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 12.9 | 7.6 |  |  |
| 300.8b | Man6 ¦ ¦ GalNAc4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 14.0 | 7.3 |  |  |
| 300.9a | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 12.5 | 6.7 |  |  |
| 300.12a | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 8.1 | 6.4 |  |  |
| 300.13b | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 8.6 | 6.6 |  |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 300.18a | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 8.2 | 8.0 |  |  |
| 300.8b | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 8.5 | 7.6 |  |  |
| 300.18c | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ GalNAc4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 9.1 | 7.3 |  |  |
| 310.3a | GlcNAc2Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 16.2 | 6.4 |  |  |
| 310.5b | Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 18.1 | 6.9 | HUK,u |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 310.8a | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 17.2 | 8.2 | HUK,t |  |
| 310.8b | Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 18.5 | 8.0 | HUK,v |  |
| 310.9a | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 16.6 | 7.3 | HUK,s |  |
| 310.12a | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 11.0 | 6.9 |  |  |
| 310.13b | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 11.8 | 7.0 |  |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 310.18a* | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 11.0 | 8.5 | HUK,g |  |
| 310.18b | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 11.7 | 8.1 | HUK,h |  |
| 310.18c* | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GalNAc4GlcNAc2 | 12.2 | 7.8 | HUK,i |  |
| 400.5a | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 10.3 | 6.6 |  |  |
| 400.6b | GlcNAc6 ¦ Man6 ¦ ¦ GalNAc4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 10.7 | 7.2 |  |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 400.13c | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ GalNAc4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 11.3 | 7.6 |  |  |
| 400.16a | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 10.5 | 9.1 |  |  |
| 400.16b | Gal4GlcNAc6 ¦ Man6 ¦ ¦ GalNAc4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 11.5 | 8.8 |  |  |
| 400.16c | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ GalNAc4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 11.9 | 8.4 |  |  |
| 410.5a | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 14.0 | 7.2 |  |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 410.6b | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 14.4 | 7.7 |  |  |
| 410.13c | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 15.1 | 8.2 |  |  |
| 410.16a | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 14.0 | 9.7 | HUK,n |  |
| 410.16b | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 15.1 | 9.4 | HUK,q |  |
| 410.16c | GalNAc4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ GalNAc4GlcNAc2 | 15.6 | 9.0 | HUK,r |  |

Sialyl PA-oligosaccharides Mono-sialyl (62)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A1-100.3 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 11.9 | 5.8 | “1A2-200.4(Serum)”-G-GN | 14 |
| 1A2-100.3 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 | 11.1 | 5.0 | “1A4-200.4(Synthesis)”-G-GN | 16 |
| 1A1-100.4 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 8.0 7.8 | 6.0 5.8 | “1A1-200.4(IgG)”-G-GN | 14 |
| 1A2-100.4 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 9.1 | 5.4 5.1 | Integrin,mD | 8 14 15 |
| 1A2-100.8 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 9.1 | 5.1 | “1A2-110.8(Integrin)”-F a little after 1A2-100.4 |  |
| 1A1-100.9 | Man6 ¦ Neu5Ac6 ¦ ¦ Man4GlcNAc4GlcNAc Gal4GlcNAc4 ¦ ¦ ¦ Man3 | 11.1 | 6.0 | “1A3-400.16(Synthesis)”-G-GN |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A2-100.9 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc Gal4GlcNAc4 ¦ ¦ ¦ ¦ Neu5Ac3 Man3 | 9.9 | 5.2 | “1A5-300.8(Synthesis)”-G-GN | 16 |
| 1A2-100.10 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc GlcNAc4 ¦ ¦ ¦ ¦ Gal3 Man3 ¦ Neu5Ac3 | 10.7 | 5.2 | “1A5-300.22(Synthesis)”-G-GN | 16 |
| 1A1-101.4 | Man6 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 9.8 | 6.0 | “1A1-201.4(Serum)”-G-GN | 14 |
| 1A2-110.3 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 | 14.7 14.5 | 5.5 | “2A2-310.18”-G-GN,-S-G-GN GN6 arm was released | 8 |
| 1A1-110.4 | Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 10.9 | 6.4 | “1A1-210.4(Serum)”-G-GN | 14 |
| 1A2-110.4 | Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.9 11.8 | 5.6 | Integrin,mG un-separable from 1A2-110.8 | 8 14 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A2-110.8 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 11.3 | 5.5 | “2A2-310.18”-G-GN,-S-G-GN GN2M3 arm was released | 8 |
| 1A1-111.4 | Man6 ¦ Fuc6 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 13.4 | 6.4 | “1A1-211.4(IgG)”-G-GN | 14 |
| 1A1-200.2 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 11.0 | 6.0 | “1A2-200.4(Serum)”-G | 14 |
| 1A2-200.2 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ GlcNAc2 | 10.9 | 5.3 | “1A4-200.4(Synthesis)”-G |  |
| 1A1-200.3 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 9.3 | 6.2 | Serum,mA | 15 |
| 1A2-200.3 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 10.8 | 5.6 5.4 | “1A3-200.4(Serum)”-G | 14 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A1-200.4* | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 9.2 9.9 | 6.9 | Integrin,mE Serum,mB IgG | 8 14 15 |
| 1A2-200.4* | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 12.0 12.2 | 6.9 | Serum,mC | 15 |
| 1A3-200.4 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 11.3 | 6.5 6.3 | Integrin,mF Synthesis | 8 14 16 |
| 1A4-200.4 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 11.7 | 6.5 6.3 | Synthesis | 16 |

Comments:

Since G.U. on ODS column for 1A2-, 1A3-, and 1A4-200.4 are very closed, cochromatography with an authentic sample is necessary.

| 1A2-200.4a1 | Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.1 | 6.3 | Fc-receptor | 17 |
| --- | --- | --- | --- | --- | --- |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A1-200.10 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 9.5 | 6.2 | “1A1-400.5(Synthesis)”-GN (incompletely) |  |
| 1A2-200.17 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc GlcNAc4 ¦ ¦ ¦ ¦ Gal3 Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 11.6 | 6.4 | “1A4-300.22(Synthesis)”-G-GN | 16 |
| 1A2-200.18 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ Gal3 | 12.9 | 6.3 | “1A6-300.22(Synthesis)”-G-GN | 16 |
| 1A1-201.3 | Man6 ¦ ¦ GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 11.9 | 6.3 | “1A1-201.4(Serum)”-G | 14 |
| 1A1-201.4 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 12.7 | 7.2 | Serum,mE |  |
| 1A1-210.2 | Neu5Ac6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 13.7 | 6.1 | Bovine thyroglobulin |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A1-210.3 | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 12.5 | 6.6 | Serum,mD IgG | 14 15 |
| 1A2-210.3 | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 14.2 | 6.0 | “1A3-210.4(Serum)”-G a little before 1A2-110.3 | 14 |
| 1A1-210.4 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 13.3 | 7.4 | Integrin,mH Serum,mF IgG | 8 14 |
| 1A2-210.4 | Neu5Ac6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 14.6 15.7 | 7.4 | Bovine thyroglobulin |  |
| 1A3-210.4 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.6 14.8 | 6.8 | Integrin,mI Serum,mG | 8 15 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A4-210.4 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 15.2 | 6.8 | Bovine thyroglobulin |  |

Comments:

Since G.U. of 1A2-, 1A3-, and 1A4-210.4 are very close, it is better to determine after fucosidase digestion. 1A31A4<1A2. Cochromatography with an authentic sample is necessary.

| 1A2-210.4a1 | Man6 ¦ ¦ Fuc6 GalNAc4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 16.3 | 6.6 | Fc-receptor | 17 |
| --- | --- | --- | --- | --- | --- |
| 1A1-211.3 | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 16.6 | 6.7 | “1A1-211.4(Serum)”-G | 14 |
| 1A1-211.4 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 18.3 | 7.7 | Serum,mH IgG | 14 15 |
| 1A2-300.2 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 12.4 | 5.7 | “1A6-300.8(Synthesis)”-G | 16 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A1-300.3 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 11.2 | 6.3 | “3A3-300.8(Fetuin)”-(2,3)S-G | 14 |
| 1A2-300.3 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.8 | 5.8 | “1A4-300.8(Synthesis)”-G | 16 |
| 1A2-300.4 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 GlcNAc2 | 12.1 | 5.8 | “1A5-300.8(Synthesis)”-G | 16 |
| 1A1-300.8 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 11.9 | 8.2 | “3A3-300.8(Fetuin)”-(2,3)S | 14 |
| 1A4-300.8 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 13.8 | 7.5 | Synthesis | 16 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A5-300.8 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 | 13.8 | 7.5 | Synthesis | 16 |
| 1A6-300.8 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 14.2 | 7.6 | Synthesis | 16 |
| 1A1-300.18 | Neu5Ac6 ¦ Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 8.2 | 8.7 | “3A3-300.18(Transferin)”-(2,3)S |  |
| 1A1-300.21 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ GlcNAc2 | 13.2 | 5.8 | “4A2-300.22(Fetuin)”-2[(2,3)S]-(2,6)S-2G | 14 |
| 1A2-300.21 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ GlcNAc2 Neu5Ac3 | 13.6 | 5.8 | “1A5-300.22(Synthesis)”-G | 16 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A1-300.22 | Gal4GlcNAc2Man6 ¦ ¦ Neu5Ac6 Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 | 15.2 | 7.5 | “4A2-300.22(Fetuin)”-2[(2,3)S]-(2,6)S | 14 |
| 1A4-300.22 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 14.0 | 7.5 | Synthesis | 16 |
| 1A5-300.22 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 | 15.7 | 7.5 | Synthesis | 16 |
| 1A6-300.22 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 | 14.5 | 7.5 | Synthesis | 16 |
| 1A1-300.23 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 | 14.8 | 6.7 | “4A2-300.22(Fetuin)”-2[(2,3)S]-(2,6)S-G | 14 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A2-300.23 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 13.9 | 6.7 | “1A4-300.22(Synthesis)”-G | 16 |
| 1A2-300.24 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ GlcNAc2 | 13.3 | 6.6 | “1A6-300.22(Synthesis)”-G | 16 |
| 1A1-400.2 | GlcNAc6 ¦ Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ GlcNAc2 | 10.5 | 7.1 | “1A2-400.16(Synthesis)”-G |  |
| 1A1-400.3 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc4Man3 ¦ GlcNAc2 | 12.0 | 7.3 | “1A3-400.16(Synthesis)”-G |  |
| 1A1-400.5 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 9.7 | 7.2 | “1A4-400.16(Synthesis)”-G |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1A2-400.16 | Gal4GlcNAc6 ¦ Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 12.2 | 9.9 | Synthesis |  |
| 1A3-400.16 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 | 13.4 | 9.9 | Synthesis |  |
| 1A4-400.16 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 10.0 | 9.9 | Synthesis |  |

Di-sialyl (50)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A1-200.4* | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 11.4 10.9 | 7.4 7.1 | Integrin,dD Serum,dA IgG | 8 14 15 |
| 2A2-200.4 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.6 | 7.0 6.4 | Synthesis | 16 |
| 2A3-200.4 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 10.8 | 7.0 6.5 | Synthesis | 16 |
| 2A4-200.4 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.4 | 6.5 5.9 | Integrin,dF1 Synthesis | 8 16 |
| 2A2-200.11 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 | 9.1 | 6.2 | “2A2-300.18(Integrin,dC1)”-G-GN |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A2-200.12 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 10.0 | 6.1 | “2A1-300.18(Integrin,dB)”-G-GN |  |
| 2A1-200.13 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 11.9 | 7.6 | “2A6-400.16(Synthesis)”-G-GN |  |
| 2A2-200.13 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 11.4 | 6.1 | “2A6-300.8(Synthesis)”-G-GN | 16 |
| 2A2-200.15 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Neu5Ac3 | 14.0 | 6.2 | “2A5-300.8(Synthesis)”-G-GN | 16 |
| 2A2-200.17 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 13.1 | 6.1 | “2A6-300.22(Synthesis)”-G-GN | 16 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A2-200.18 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ Gal3 ¦ Neu5Ac3 | 15.2 | 6.1 | “2A5-300.22(Synthesis)”-G-GN | 16 |
| 2A1-201.4 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 14.6 | 7.4 | “2A1-211.4(Serum; IgG)”-F | 14 |
| 2A1-210.4 | Neu5Ac6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 13.7 14.3 | 7.6 | Integrin,dH Serum,dC IgG | 8 14 15 |
| 2A4-210.4 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 17.1 16.5 | 6.9 6.3 | Integrin,dJ Rabies virus | 8 |
| 2A2-210.11 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ Man3 | 11.3 | 6.5 | “2A2-310.18(Integrin,dG1)”-G-GN |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A2-210.12 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.5 | 6.3 | “2A1-310.18(Integrin,dF)”-G-GN |  |
| 2A1-211.4 | Neu5Ac6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ GlcNAc4-Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 18.2 | 7.8 | Serum,dD IgG | 14 15 |
| 2A1-300.5 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 12.6 | 7.4 | “2A1-300.8(Serum)”-G | 14 |
| 2A2-300.5 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 14.7 | 6.3 | “2A4-300.8(Synthesis)”-G | 16 |
| 2A2-300.6 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ GlcNAc2 | 14.0 | 6.3 | “2A5-300.8(Synthesis)”-G | 16 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A1-300.8 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 13.1 | 8.3 | Serum,dB | 15 |
| 2A4-300.8 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.1 | 7.1 | Synthesis | 16 |
| 2A5-300.8 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 | 15.1 | 7.1 | Synthesis | 16 |
| 2A6-300.8 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 14.2 | 7.1 |  | 16 |
| 2A2-300.9 | GlcNAc2Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 13.9 | 6.3 | “2A6-300.8(Synthesis)”-G | 16 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A2-300.13 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 9.4 | 6.4 | “2A1-300.18(Integrin,dB)”-G |  |
| 2A2-300.14 | GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 9.6 | 6.6 | “2A3-300.18(Integrin,dC2)”-G |  |
| 2A2-300.15 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ GlcNAc2 | 9.2 | 6.4 | “2A2-300.18(Integrin,dC1)”-G |  |
| 2A1-300.18 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 9.5 | 7.2 | Integrin,dB | 8 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A2-300.18 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 9.2 10.0 | 7.3 | Integrin,dC1 | 8 |
| 2A3-300.18 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 10.0 | 7.3 | Integrin,dC2 | 8 |
| 2A4-300.18 | Gal4GlcNAc6 ¦ Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 7.9 | 8.7 | “3A4-300.18(Transferin)”-(2,3)S |  |
| 2A4-300.22 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.4 | 7.1 | Synthesis | 16 |
| 2A5-300.22 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 | 17.1 | 7.2 | Synthesis | 16 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A6-300.22 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 16.2 | 7.1 | Synthesis | 16 |
| 2A2-300.23 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.9 | 6.4 | “2A6-300.22(Synthesis)”-G | 16 |
| 2A2-300.24 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Neu5Ac3 ¦ GlcNAc2 | 15.7 | 6.3 | “2A5-300.22(Synthesis)”-G | 16 |
| 2A1-300.31 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Fuc3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 12.9 | 9.1 | “3A2-300.31(Serum)”-(2,3)S | 14 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A1-310.8 | Neu5Ac6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 16.8 16.9 | 9.1 | “3A2-310.8(Transferin)”-(2,3)S |  |
| 2A2-310.13 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.4 | 6.7 | “2A1-310.18(Integrin,dF)”-G |  |
| 2A2-310.14 | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.2 | 6.9 | “2A3-310.18(Integrin,dG2)”-G |  |
| 2A2-310.15 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ GlcNAc2 | 11.8 | 6.8 | “2A2-310.18(Integrin,dG1)”-G |  |
| 2A1-310.18 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.3 | 7.5 | Integrin,dF2 | 8 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A2-310.18 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 12.6 | 7.6 | Integrin,dG1 | 8 |
| 2A3-310.18 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.6 | 7.6 |  | 8 |
| 2A1-310.31 | Neu5Ac6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Fuc3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 16.3 16.4 | 9.8 | “3A2-310.31(Transferin)”-(2,3)S |  |
| 2A1-400.6 | GlcNAc6 ¦ Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 10.8 | 8.3 | “2A5-400.16(Synthesis)”-G |  |
| 2A1-400.8 | GlcNAc6 ¦ Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 12.1 | 8.5 | “2A6-400.16(Synthesis)”-G |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2A5-400.16 | Gal4GlcNAc6 ¦ Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 11.5 | 10.0 | Synthesis |  |
| 2A6-400.16 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 12.0 | 10.1 | Synthesis |  |

Tri-sialyl (38)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 3A1-300.8 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 15.3 | 8.5 | Serum,tc Transferin,Tri-L | 15 |
| 3A2-300.8* | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 13.1 | 8.0 | Serum,tB Transferin,Tri-K | 15 |
| 3A3-300.8* | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 13.0 | 7.6 | Fetuin |  |
| 3A4-300.8 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.0 15.4 | 6.8 | Synthesis | 14 |
| 3A1-300.18 | Neu5Ac6 ¦ Gal4GlcNAc6 ¦ Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 8.2 | 8.8 | Transferin,Tri-A |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 3A2-300.18 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 9.8 | 6.8 | Integrin,tA | 8 |
| 3A3-300.18 | Neu5Ac6 ¦ Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 9.1 | 8.1 | Transferin,Tri-D |  |
| 3A4-300.18 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 ¦ Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Ac6 Man3 ¦ ¦ Gal4GlcNAc2 | 9.0 | 8.4 | Transferin,Tri-C |  |
| 3A5-300.19 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Neu5Ac3 | 11.0 | 7.4 | “3A5-410.16(Integrin)”-F-G-GN |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 3A3-300.20 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.8 | 7.1 | “3A3-410.16(Integrin)”-F-G-GN |  |
| 3A1-300.22 | Neu5Ac6Gal4GlcNAc2Man6 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 13.4 | 7.9 | “4A1-300,22(Fetuin)”-(2,3)S | 14 |
| 3A4-300.22 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 17.8 | 6.8 | Synthesis | 16 |
| 3A2-300.31 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ ¦ Neu5Ac3 ¦ ¦ Fuc3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 13.0 13.6 | 8.4 8.7 | Serum,tA Transferin,Tri-J | 15 |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 3A1-310.8 | Neu5Ac6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 19.6 | 8.7 | Serum,tE | 15 |
| 3A2-310.8 | Neu5Ac6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 16.7 17.3 | 8.4 8.8 | Serum,tD | 15 |
| 3A4-310.8 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 20.5 19.6 | 8.0 7.3 | Integrin,tF | 8 |
| 3A2-310.18 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.6 12.2 | 7.5 7.1 | Integrin,tB | 8 |
| 3A5-310.19 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Neu5Ac3 | 13.3 | 7.5 | “3A5-410.16(Integrin)”-G-GN |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 3A3-310.20 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.7 | 7.2 | “3A3-410.16(Integrin)”-G-GN |  |
| 3A2-310.31 | Neu5Ac6Gal4GlcNAc2Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ ¦ Neu5Ac3 ¦ ¦ Fuc3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 16.7 | 9.5 | Transferin,Tri-M |  |
| 3A6-400.12 | GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.2 | 7.6 | “3A6-410.16(Integrin)”-F-G |  |
| 3A8-400.12 | GlcNAc6 ¦ Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 13.5 | 9.5 | “3A8-400.16(Synthesis)”-G |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 3A4-400.13 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 11.6 | 7.5 | “3A4-410.16(Integrin)”-F-G |  |
| 3A3-400.14 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.2 | 7.4 | “3A3-410.16(Integrin)”-F-G |  |
| 3A5-400.15 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ GlcNAc2 | 11.8 | 7.5 | “3A5-410.16(Integrin)”-F-G |  |
| 3A3-400.16 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 11.5 | 8.1 | “3A3-410.16(Integrin)”-F |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 3A4-400.16 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.1 | 8.2 | “3A4-416.16(Integrin)”-F |  |
| 3A5-400.16 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 | 12.5 | 8.1 | “3A5-410.16(Integrin)”-F |  |
| 3A6-400.16 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 12.5 | 8.1 | “3A6-410.16(Integrin)”-F |  |
| 3A8-400.16 | Gal4GlcNAc6 ¦ Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc4Man3 Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 13.5 | 10.2 | Synthesis |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 3A6-410.12 | GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.7 | 7.7 | “3A6-410.16(Integrin)”-G |  |
| 3A4-410.13 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.0 | 7.5 | “3A4-410.16(Integrin)”-G |  |
| 3A3-410.14 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.4 | 7.6 | “3A3-410.16(Integrin)”-G |  |
| 3A5-410.15 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ GlcNAc2 | 14.9 | 7.6 | “3A5-410.16(Integrin)”-G |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 3A3-410.16 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 14.7 | 8.3 | Integrin,tC | 8 |
| 3A4-410.16 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.2 | 8.3 | Integrin,tD | 8 |
| 3A5-410.16 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 | 15.9 | 8.5 | Integrin,tE1 | 8 |
| 3A6-410.16 | Gal4GlcNAc6 ¦ Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 15.9 | 8.5 | Integrin,tE2 | 8 |

Tetra-sialyl (4)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 4A1-300.22 | Neu5Ac6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Neu5Ac3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 13.8 | 7.8 | Fetuin | 14 |
| 4A2-300.22 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ ¦ Neu5Ac3 Man4GlcNAc4GlcNAc Neu5Ac6 ¦ ¦ ¦ GlcNAc4Man3 ¦ ¦ Gal3 ¦ ¦ ¦ Neu5Ac3 ¦ Neu5Ac6 ¦ ¦ ¦ Gal4GlcNAc2 | 13.4 | 7.4 | Fetuin | 14 |
| 4A2-400.16 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 11.5 | 8.7 7.6 | Integrin,teA | 8 |
| 4A2-410.16 | Gal4GlcNAc6 ¦ ¦ Neu5Ac3 Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ ¦ Man4GlcNAc4GlcNAc Neu5Ac3 ¦ ¦ Gal4GlcNAc4Man3 ¦ ¦ Neu5Ac3 ¦ Gal4GlcNAc2 ¦ Neu5Ac3 | 14.3 | 9.0 7.8 | Integrin,teB | 8 |

N-glycolyl neuraminic acid (Neu5Gc)-containing oligosaccharides.

Mono-sialyl (12)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1G1-100.3 | Neu5Gc6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 9.5 | 6.0 | “1G1-210.2”-F-GN |  |
| 1G1-100.4 | Man6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Gc6 Man3 ¦ ¦ Gal4GlcNAc2 | 6.2 | 6.1 | “1G1-210.3”-F-GN |  |
| 1G1-110.3 | Neu5Gc6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 | 11.8 | 6.4 | “1G1-210.2”-GN |  |
| 1G1-110.4 | Man6 ¦ Fuc6 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Gc6 Man3 ¦ ¦ Gal4GlcNAc2 | 8.9 | 6.5 | “1G1-210.3”-GN |  |
| 1G1-200.2 | Neu5Gc6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 8.9 | 6.2 | “1G1-210.2”-F |  |
| 1G1-200.3 | Man6 ¦ ¦ GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Gc6 Man3 ¦ ¦ Gal4GlcNAc2 | 7.5 | 6.4 | Mouse IgG |  |

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 1G1-200.4 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Gc6 Man3 ¦ ¦ Gal4GlcNAc2 | 8.0 | 7.3 | “1G1-210.4”-F |  |
| 1G2-200.4 | Neu5Gc6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 9.7 | 7.2 | “1G2-210.4”-F |  |
| 1G1-210.2 | Neu5Gc6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ GlcNAc2 | 11.1 | 6.7 | Mouse IgG |  |
| 1G1-210.3 | Man6 ¦ ¦ Fuc6 GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Gc6 Man3 ¦ ¦ Gal4GlcNAc2 | 10.0 | 6.8 | Mouse IgG |  |
| 1G1-210.4 | Man6 ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Gc6 Man3 ¦ ¦ Gal4GlcNAc2 | 10.8 | 8.1 7.7 | Mouse IgG |  |
| 1G2-210.4 | Neu5Gc6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Man3 ¦ Gal4GlcNAc2 | 12.0 | 7.7 | Mouse IgG Bovine thyroglobulin |  |

Di-sialyl (3)

| Code No. | PA-Oligosaccharide | Glucose Unit |  |  | Source (Comment) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ODS | Amide |  |  |
| 2G1-200.4 | Neu5Gc6 Man6 ¦ ¦ ¦ Gal4GlcNAc2 ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Gc6 Man3 ¦ ¦ Gal4GlcNAc2 | 7.5 | 7.8 | Mouse IgG |  |
| 2G3-200.4 | Man6 ¦ ¦ Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc Neu5Gc3 ¦ ¦ Neu5Gc6 Man3 ¦ ¦ Gal4GlcNAc2 | 8.7 | 7.4 | Mouse fibrinogen |  |
| 2G1-210.4 | Neu5Gc6 Man6 ¦ ¦ ¦ Fuc6 Gal4GlcNAc2 ¦ ¦ Man4GlcNAc4GlcNAc ¦ ¦ Neu5Gc6 Man3 ¦ ¦ Gal4GlcNAc2 | 9.8 | 8.2 | Mouse IgG |  |